9.3.7 Dwelling house code

9.3.7.1 Application

This code applies to assessing a material change of use or building work if:

self-assessable or assessable development where this code is an applicable code identified in the assessment criteria column of a table of assessment for a material change of use (section 5.5), a neighbourhood plan (section 5.9) or an overlay (section 5.10); or

impact assessable development for a dwelling house if not on a small lot or a use of a similar nature.

When using this code, reference should be made to section 1.5 and section 5.3.3.

Note—Where a dwelling house on a small lot, the Dwelling house (small lot) code applies.

Note—Where the land is identified in an overlay such as the Bushfire overlay map, Flood overlay map, Landslide overlay map, Significant landscape trees overlay map or Waterway corridors overlay map, additional provisions relating to that overlay also apply. For example, minimum floor levels for a dwelling house on a site subject to certain types of flooding, are identified in the Flood overlay code.

Note—Where the site is located may be subject to a zone, zone precinct, neighbourhood plan or overlay, these may vary the outcomes identified in this code and to the extent that these vary, those outcomes prevail.

Note—Preliminary approvals or development permits for other aspects of development can vary the outcomes of this code and to the extent that these vary, those outcomes prevail.

Editor’s note—Depositing or allowing sediment or other water contaminants to wash or move into roadways, stormwater pipes and waterways without taking appropriate steps to prevent this occurring, is a breach of the Environmental Protection Act 1994. Penalties apply including fines and prosecution. Additional information, including best practice guidelines for controlling stormwater pollution from building sites, can be found in the SEQ Healthy Waterways - Water by Design program’s Erosion and Sediment Control factsheets.

Editor’s note—For a proposal to be self-assessable, it must meet all the self-assessable outcomes of this code and any other applicable code. Where it does not meet all self-assessable outcomes, the proposal becomes assessable development and a development application is required. Where a development application is triggered, only the specific acceptable outcomes that the proposal fails to meet need to be assessed against the corresponding assessable acceptable outcomes or performance outcomes. Other self-assessable outcomes that are met are not assessed as part of the development applications.

9.3.7.2 Purpose

- The purpose of the Dwelling house code is to assess the suitability of development to which this code applies.

The purpose of the code will be achieved through the following overall outcomes:

- Development ensures that a dwelling house, including a habitable building on the site, is occupied by the same single household.

Development for a dwelling house, including a secondary dwelling or domestic outbuilding, incorporates design and siting to allow reasonable access to daylight and sunlight for neighbouring dwellings and private open space.

Development of a dwelling house provides sufficient and safe vehicle access and parking for residents.

Development ensures that a dwelling house, including a secondary dwelling or domestic outbuilding, does not cause adverse drainage impacts or flooding of upstream, downstream or adjoining land.

Development ensures that the siting of a dwelling house and any built to boundary walls do not negatively impact on the privacy and amenity of adjoining residents.

9.3.7.3 Assessment criteria

The following table identifies the assessment criteria for self-assessable and assessable development.

Table 9.3.7.3—Criteria for self-assessable and assessable development

| Performance outcomes | Acceptable outcomes |
| --- | --- |
| PO1 Development ensures that a building other than a dwelling house on the site: is used for a domestic residential purpose, in conjunction with and subordinate to the dwelling house on the same site; is smaller in size and scale than the dwelling house; has the appearance of a building ancillary to the dwelling house; is occupied by members of the same household who occupy the dwelling house. | AO1.1 Development comprises not more than one dwelling house and one secondary dwelling, occupied by one household comprising: - 1 person maintaining a household; or 2 or more persons related by blood, marriage or adoption; or not more than 5 persons, not necessarily related by blood, marriage or adoption; or not more than 5 persons under the age of 18 and not necessarily related by blood, marriage or adoption, together with 1 or 2 adult persons who have care or control of them. |
|  | AO1.2 Any development for a secondary dwelling is: - a maximum of 80m2 in gross floor area; located within 20m of the dwelling house; occupied by 1 or more members of the same household as the dwelling house. |
| PO2 Development has a building height that: - is consistent with the building height of dwelling houses prevailing in the immediate vicinity; does not unduly overshadow adjoining dwelling houses and their associated private open space in terms of access to sunlight and daylight. Note–In interpreting PO2, the term ‘prevailing in the immediate vicinity’ means the building height of more than 50% of the dwelling houses in the same zone as the subject site and within 35m of any point of the street frontage of the subject site. | AO2 Development in the: - Low density residential zone, Character residential zone, 2 storey mix zone precinct of the Low–medium density residential zone, 2 or 3 storey mix zone precinct of the Low–medium density residential zone, Rural residential zone, Environmental management zone, Rural zone or Emerging community zone results in a maximum building height of 9.5m and: 2 storeys; or 1 storey if the development also includes a space that is situated between one floor level and the floor level next above, or if there is no floor above, the ceiling or roof above that contains only a bathroom, shower room, laundry, water closet, or other sanitary compartment; Up to 3 storeys zone precinct of the Low–medium density residential zone or in the Medium density residential zone results in a maximum building height of 11.5m and: - 3 storeys; or 2 storeys if the development also includes a space that is situated between one floor level and the floor level next above, or if there is no floor above, the ceiling or roof above that contains only a bathroom, shower room, laundry, water closet, or other sanitary compartment. Editor's note—This acceptable outcome is only for the maximum building height. Side boundary setbacks are provided in accordance with the Queensland Development Code; which vary according to the height of the building. |
| PO3 Development ensures that residents’ vehicles are accommodated on site. | AO3 Development provides a minimum number of on-site parking spaces comprising: - 1 car parking space for the dwelling house; 1 car parking space for any secondary dwelling on the same site. |
| PO4 Development ensures that the location and design of a dwelling house, secondary dwelling or domestic outbuilding does not expose surrounding properties to additional adverse impacts of overland flow. | AO4 Development and associated site works, including filling and excavation, are designed and constructed to ensure overland flow is not worsened, impeded, or otherwise diverted to adversely affect other properties. |
| PO5 Development for a dwelling house, secondary dwelling or domestic outbuilding, including associated site works such as retaining walls, filling and excavation ensures that if a surface- or roof-water drainage system connection is required through an adjoining property, the surface- or roof- water drainage system is managed to prevent water seepage, concentration of run-off or ponding on an adjoining property. Note—The Queensland Development Code outlines requirements for surface- and roof-water drainage systems for Class 1 buildings and Class 10 buildings and structures where a surface- or roof-water drainage connection is not required through an adjoining property. | AO5 Development for a dwelling house, secondary dwelling or domestic outbuilding, including associated site works such as retaining walls, filling and excavation ensures that if a surface- or roof- water drainage system connection is required through an adjoining property, the owner of the adjoining property has provided written permission for the connection. |
| If for a site with an approved building envelope plan or development footprint plan |  |
| PO6 Development is sited to complement and be consistent with the form and character of the local area, having regards to: - values of the locality expressed in overlays, neighbourhood plans, zones and infrastructure; location of buildings on adjoining sites, ensuring sufficient separation of buildings to minimise impacts on residential amenity and privacy; the impact of slope. | AO6 Development has a footprint that fits entirely within the approved building envelope or development footprint plan. |
| If in the Environmental management zone, Rural zone, Rural residential zone or in a very-low density residential area identified in a neighbourhood plan |  |
| PO7 Development does not adversely impact on scenic features. | AO7 Development maximum building height is located a minimum of 15m vertically below any ridgeline within 150m horizontal distance of the dwelling house. |